Main Technical Data

TABLE OF CONTENTS	PAGE

1.0	Main Dimensions	2

2.0	Weights	2

3.0	Main Drive	2

4.0	Speeds	3

5.0	Axles and Tyres	3

6.0	Turning Radius	3

7.0	Wind forces	4

8.0	Utilization of Tipping Load	4

9.0	Crane and Mechanism Group Classification	4

10.0	Lighting	5

11.0	Painting	5

12.0	Spreader Systems for ISO Containers	6

1.0	Main Dimensions

Length of the chassis without stabilizer pads	approx.	17,0 m

Width of chassis without stabilizer pads	approx.	8,7 m

Size of stabilizer pads (other sizes on request)		4 x 2,0 x 4,5 m

Propping base		12,0 x 13,0 m

Tail radius		7,2 m

Height of the boom fulcrum point 	approx.	21,8 m

Viewing height of crane operator	approx.	27,9 m

Boom length		50,5 m

Max. outreach		50,0 m

Min. outreach		11,0 m

Lifting height above quay	11 m – 40 m radius	40,0 m

40 m – 45 m radius	40,0 m

45 m – 50 m radius	29,0 m

Lifting height below quay	12,0 m

# 2.0	Weights

Counterweight		114,0 t

Total weight of fully rigged crane	approx.	435,0 t

# 3.0	Main Drive

Type of drive system	Diesel - electric

## 3.1	Diesel Engine

## Manufacturer						Cummins

## Type							QST30-G1 / EPA

## Combustion type					Diesel

## Cooling						Water

## Power							701 kW at 1500 rpm

## Number of cylinder					12

## Consumption						max. 205g / kWh

## 3.2	Fuel Tank

Capacity of fuel tank	approx.	7300 l

Possible operating hours	about 	170 h

# 4.0	Speeds

## 4.1	Hoisting

- heavy lift operation		100,0 t	15 m/min

60,0 t	25 m/min

40,0 t	34 m/min

26,0 t	40 m/min

- standard operation		63,0 t	25 m/min

40,0 t	40 m/min

13,0 t	70 m/min

## 4.2	Slewing

- speed at standard operation	up to 1,35 rpm

- speed at heavy lift operation	up to 0,6 rpm

- maximum speed boom head	180 m/min

## 4.3	Luffing

- standard operation	0 - 60 m/min

- heavy lift operation	0 - 27 m/min

4.4	Travelling 	0 - 80 m/min

4.5	Climbing ability 	max. 6 %

# 5.0	Axles and Tyres

Number of axles	7

- steerable	6

- driven		3

Size of tyre	14.00-24

Total number of tyres	28

# 6.0	Turning Radius

Inner turning radius	approx. 6,0 m

Outer turning radius	approx. 14,8 m

# 7.0	Wind forces

-crane in operation	up to 	24 m/s

(9 Beaufort)

-crane out of operation	up to 	46 m/s

(14 Beaufort)

# 8.0	Utilization of Tipping Load

General cargo operation	< 66 %

Heavy load operation	66 % / 75 %

Grab operation	  < 50 %

# 9.0	Crane and Mechanism Group Classification

Classification according to:	FEM 1.001

1998

## 9.1	Steel structure

Heavy lift operation	100 t	A3

General cargo operation	63 t	A5

Grab operation	40 t	A8

## 9.2	Machinery

Hoisting

- standard operation	63 t	M6

- heavy lift operation	100 t	M3

- grab operation	40 t	M7

Slewing

- standard operation	63 t	M6

- heavy lift operation	100 t	M5

- grab operation	40 t	M7

Luffing

- standard operation	63 t	M6

- heavy lift operation	100 t	M5

Travelling	M4

# 10.0	Lighting

Metal vapour light under boom head	1 x 2000 W

Floodlight (Halogen) under boom	2 x 2000 W

Floodlight (Halogen) tower front	2 x 2000 W

Floodlight at the rear of the superstructure	1 x 2000 W

Floodlight at the tower sides (optional) 	2 x 2000 W

Aviation warning lights (optional) 	1 - 2

Yellow flasher lights	2

# 11.0	Painting

All load bearing parts are blast cleaned and painted to an approved specification taking into consideration the corrosive saline atmosphere in ports. The steel surface will be blast cleaned to preparation class SA 2.5.

Base paint:		2-component epoxy-resin			60 m

Intermediate coat:	2-component epoxy-resin			60 m

Final paint:		2-component acryl-polyurethane-resin	50 m

Total thickness of paintwork is minimum 170 m according to EN ISO 12944-5.

Gottwald standard

Chassis	RAL 7001 grey

Rims, outriggers, staircases	RAL 5015 blue

Superstructure with housing	RAL 5015 blue

Tower complete with staircases	RAL 7001 grey

Luffing cylinder	RAL 5015 blue

Boom (Jib)	RAL 5015 blue

Rope pulleys (Jib-head)	RAL 1003 yellow

Drivers cabin	RAL 1013 white

Tower cabin	RAL 1013 white

Counter weight	RAL 5015 blue

Warning stripes outriggers black / yellow	RAL 9005/RAL 1003

Warning stripes counterweight black / yellow	RAL 9005/RAL 1003

LOGO, both sides of superstructure	please add specimen

# 12.0	Spreader Systems for ISO Containers

Spreader type	Capacity under	Weight

container type	spreader

(t)	(t)

Fully Automatic Telescopic Spreader for

20’ - 40’ container	41	9,0

Fully Automatic Fix Spreader (with suspension tower) for

20' container	25	4,0

40' container	35	6,5

Fully Automatic Telescopic Lightweight Spreader for

20’ - 40’ container	35	6,9

with twin lift equipment 2 x 20’ container	2 x 25 	10,7

Manual Fix Spreader for

20' container	32	1,2

40' container	36	2,2

Semi automatic Fix Spreader with Set-Down-Automatic for

20' container	32	1,2

40' container	36	2,3

Other spreader sizes on request!

Subject to technical modification without prior notice!