____________________________________________________________

MATERIAL SAFETY DATE SHEET

SULPHURIC ACID

General

Synonyms: oil of vitriol, mattling acid, vitriol, battery acid, dipping acid, electrolyte acid, vitriol brown oil, sulphuric acid

Molecular formula: H2SO4

CAS No: 7664-93-9

EC No: 231-639-5

EC index No: 016-020-00-8

Physical data

Appearance: Colourless oily liquid

Melting point: -2 C

Boiling point: 327 C

Specific gravity: 1.84

Vapour pressure: Flash point:

Explosion limits:

Autoignition temperature:

Water solubility: miscible in all proportions

Stability

Stable, but reacts with moisture very exothermically, which may enhance its ability to act as an oxidizing agent. Substances to be avoided include water, most common metals, organic materials, strong reducing agents, combustible materials, bases, oxidizing agents. Reacts violently with water - when diluting concentrated acid, carefully and slowly add acid to water, not the reverse. Reaction with many metals is rapid or violent, and generates hydrogen (flammable, explosion hazard).

Toxicology

Extremely corrosive, causes serious burns. Highly toxic. Harmful by inhalation, ingestion and through skin contact. Ingestion may be fatal. Skin contact can lead to extensive and severe burns. Chronic exposure may result in lung damage and possibly cancer.

Toxicity data

IHL-RAT LC50 0.51 mg/l

UNR-MAN LDLO 135 mg kg-1

ORL-RAT LD50 2140 mg kg-1 (25% solution)

IHL-MUS LC50 320 mg m -3 / 2h

IHL-GPG LC50 18 mg m-3

Risk phrases

R35 R36 R37 R38 R49.

Transport information

UN No 1830. IMDG class 8. Packing group II. IATA 1830. UK transport category 2.

Personal protection

Safety glasses or face mask; acid-resistant gloves. Suitable ventilation. In the UK use of this material must be assessed under the COSHH regulations.

Safety phrases

S23 S30 S36 S37 S39 S45.