# Examining the Gravity of Take-Ups.          Eur Ing John Gateley

## England

### Summary

A simple and effective Gravity Take Up system has evolved which replaces the earlier pulley carriages with rope sheaves directly fitted to the shafts. Rolling guidance is automatically available in both horizontal and vertical applications. Standard maintenance operations such as belt repair, splicing and bearing replacement are greatly enhanced by the superior counterweight manipulation available.

### Present GTU Drawbacks

Gravity take-ups are the preferred method of adjusting the belt line to match elongations caused by tension fluctuations along the belt. When it comes to the detail design stage the GTU’s become very irksome indeed and much expenditure is involved with drawing guides, take-up trolleys/carriages and tracks. Steelwork becomes complicated mainly because the take-up mechanism has a larger working envelope than the conveyor pulleys. In horizontal take up situations the addition of a trolley to carry the pulley has unduly increased the self-weight of the moving mass. More importantly the provision of a carriage has highlighted the complexity involved in aligning the counterbalance forces to the belt tensions, to keep the carriage running flat on the tracks.

### TrakSheaves

To reduce the working envelope of the mechanism Anglonesian have invented and developed the TrakSheave Take up carriage. Simple rope sheaves are fitted to the pulley shafts and the counterweight(s) are slung on the standing part of the rope and the bight is either anchored or fastened to a winch for counterweight adjustment.

### Improved Guidance

In previous take-up systems the counterweight guidance/containment usually involved rubbing a heavy mass against the guides. If roller guidance was provided this was at additional expense. Now with the TrakSheave equipment the roller guidance is integral with the sheaves. There is substantial chamfer relief on the sheave faces which allows them to run in simple rolled steel angle guides with complete accuracy. This guidance method is equally suited for both vertical and horizontal take up schemes. Rope parts are contained in the voids at the angle apices and make no contact with the runners. Accordingly there is no need to line or face the guides with low friction, wear resistant , call it what you will, material.

Figure 1; Typical vertical arrangement

#### Faster and Safer Maintenance

With the counterweight location controlled by the working rope the belt maintenance workload is reduced. Reference to figures 1 or 3 show that the winch can lower the counterweight to ground safely, quickly and without disturbance to the belt. Note also that the counterweight operating envelope can be adjusted to suit the plant layout requirements, either down near the ground or elevated to suit a platform height.

Because the reeving is so simple the winch location is open ended and the item can be placed where it is most useful. With sufficient spare rope on the winch drum the take up pulley can be hoisted clear off the belt loop by unhitching the rope from the fully lowered counterweight and wrapping the rope under the pulley sheave(s). With the bight secured to nearby steelwork the take up pulley can be lifted clear by winding in the installed winch. No external rigging is required to slacken the belt for repair and splicing operations.

A horizontal take up arrangement is shown in figure 2. Here the usefulness of the pulley sheaves is even more prevalent. As well as eliminating the take up trolley the system merely utilises equipment already available in existing take up systems, namely the rope(s) and winch.

.

#### OEM & Retrofit Situations

In a retrofit scenario the TrakSheave equipment can be fitted alongside a normal splicing operation after some minor preliminary structural modifications that would not interfere with conveyor normal running. This work involves replacing the original rubbing guides with the TrakSheave guides and fitting  the winch, if necessary. Note that only the rope length within the travel range flexes. Accordingly it is recommended that the counterweight position is shifted periodically to comply with good reeving practice. This is easy to achieve with the arrangement shown in figure 3 whereas the arrangements shown in figures 1 and 2 will require spare rope, approximately 1 metre, available at the fixed ends. Sheaves are bored to match the bearings used in existing plummer blocks.

Figure 2: Typical horizontal installation

Figure 3 : Simplest arrangement for a vertical GTU.