Dear Sir,

- Twin Boom Stacker (TBS) is in service from the past 8 years

- It is linear stacker unloading only iron ore fines.

- There are 12nos of boogie wheels out of which 8nos are driven by motor i.e. for every two wheels one motor-gear box.

- There are four main columns; each column is having two wheels.

- All drives are of locomotive wheel set.

- The distance between the rails is 7000mm

- The distance between  each column length wise 7500mm

- The rails are mounted on the concrete bed and bolts are anchored.

- One continuous rail each joint welded together

- Initially there was gap between the rails and later we welded to counter expansion problem.

- Stacker movement is controlled by end limit switch like over travel forward and backward relays and all are inline

Major Jobs carried out in Twin Boom stacker.

- All the four legs have been replaced one by one, because there was the crack in all the main Columns till the inclined position.

- Very recently we have changed two main columns.

- After that two of the LT shafts sheared with in 15 days.

- We found that 34mm difference in Diagonals and that this mite be the cause for shaft shear.

7012

7004

7

4

9

0

7

5

0

4

9868

9904

Present dimensions of the Twin Boom Stacker